# My House Exam Checklist

# Mark a checkmark ✓ for items that appear to be in good condition.

# Make notes for any items that need repair or replacement.

Please Note: This checklist is a starting point, not a replacement for a professional home inspection.

1

| Checklist |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | GROUNDS. |  |  |  |
|  | Proper grading, ground sloping, draining water away from house. 1-inch per Ft for first 6 Ft |  |  |  |
|  | No branches or bushes touching house or overhanging the roof. |  |  |  |
|  | Downspouts from roof directed away from home 4 feet at minimum. |  |  |  |
|  | Driveways, sidewalks, patios, entrance landings in good condition. |  |  |  |
|  | Railings on stairs and decks are safe and secured. |  |  |  |
|  | Misc exterior structures (fences, sheds, decks, retaining walls) in good condition. |  |  |  |
|  | EXTERIOR. |  |  |  |
|  | Siding appears in good condition, no holes or damage. |  |  |  |
|  | Adequate clearance between ground and siding materials. |  |  |  |
|  | Foundation in good condition - appears straight with no significant cracks. |  |  |  |
|  | Sides of house appear straight, not bowed or sagging |  |  |  |
|  | Brick or stone siding: no cracks in joints, no broken material, paint in good condition. |  |  |  |
|  | Window and door frames appear level and have no large gaps. |  |  |  |
|  | ROOF. |  |  |  |
|  | Gutters: no decay or rust, joints sealed, attached securely to home, no bending or sagging, no sections of gutter or downspout missing, gutters clean |  |  |  |
|  | Shingles: no curling shingles, no visible nails or nail holes, no loss of granules in gutters, no broken, damaged or missing shingles, no more than two layers of shingles. |  |  |  |
|  | Chimneys: straight, properly flashed, no evidence of damaged bricks or cracked joints, cement cap in good condition, chimney cap present. |  |  |  |
|  | Proper Flashing & sealing around roof penetrations. |  |  |  |
|  | Soffits and fascia: no decay or nests blocking/vents are clean and not painted over. |  |  |  |
|  | INTERIOR ROOMS. |  |  |  |
|  | Floors, walls and ceilings appear straight and level. |  |  |  |
|  | No stains on floors, walls or ceilings. |  |  |  |
|  | No significant cracks in walls or ceilings. |  |  |  |
|  | Lights and switches operate properly, any appliances are tested and working properly. |  |  |  |
|  | Adequate number of three pronged electrical outlets, cover plates in good conditon, no exposed wiring. |  |  |  |
|  | BATHROOMS. |  |  |  |
|  | Run Sink & Tub simultaneuously for 10 minuets. Watch for leaks & proper drainage. |  |  |  |
|  | Plumbing and cabinet/floor under sink in good condition, no stains or leaks. |  |  |  |
|  | Exhaust fan in working condition & vents outside of home. |  |  |  |
|  | Proper shower pressure & all attachments properly secured & sealed. |  |  |  |
|  | GFCI within 6ft of water source. |  |  |  |
|  | Toilet flushes properly & is stable, no rocking or water stains around base. |  |  |  |

2

| Checklist |  |  |  |
| --- | --- | --- | --- |
|  | KITCHEN. |  |  |
|  | Run plumbing (sink, dishwasher etc..) for 10 minutes. – Check often for overflow, leaks & proper drainage! |  |  |
|  | Check for GFCI outlets within 6ft of water sources. |  |  |
|  | Cabinets in good condition: doors and drawers operate properly. |  |  |
|  | Floor in cabinet under sink solid, no stains or decay, no leaks , no rust on pipes under sinks. |  |  |
|  | Test all appliances if applicable ( stove, microwave, garbage disposal, etc... ) |  |  |
|  | BASEMENT. |  |  |
|  | No wall stains, no flaking, no efflorescence or signs of moisture. |  |  |
|  | No major cracks, 1/8th Inch or larger for verticle cracks , horizontal cracks & step crackin should be reviewed by a foundation specialist. |  |  |
|  | Visible supply lines: Properly supported. No bending, no leaking, no corrosion. |  |  |
|  | Visible drains: No damage, properly sloping to main drain, no signs of leaks. |  |  |
|  | WATER HEATER. |  |  |
|  | No signs of rust or damaged water lines. |  |  |
|  | Gas vent to exterior is in good condition and sealed properly to keep fumes out of home. |  |  |
|  | No damage or leaks on or around water heater. |  |  |
|  | Adjust water temp gauge as needed. 120 degrees should be the lowest (to prevent bacteria growth in the water heater) and 140 the highest (to prevent scolding & burns). |  |  |
|  | 3 |  |  |
|  | ELECTRICAL. |  |  |
|  | No rust or water on electrical panel, IF SO, DO NOT TOUCH & HAVE IT EVALUATED BY AN ELECTRICIAN. |  |  |
|  | No scorch marks or exposed wires. IF SO, DO NOT TOUCH & HAVE IT EVLALUATED BY AN ELECTRICIAN. |  |  |
|  | Adequate capacity 200 amps if necessary. |  |  |
|  | Panel brand and breakers match. No open breaker spots or damaged breakers. |  |  |
|  | HEATING & COOLING. |  |  |
|  | Inside of unit is rust, water & scorch mark free. |  |  |
|  | Vent to Chimney or exterior is sealed and no holes/openings. |  |  |
|  | No rust in or around unit. |  |  |
|  | Air Filter Clean? |  |  |
|  | Test that heat works by using thermostat. Close unit properly prior to turning heat on. |  |  |
|  | If it is above 65 degrees Fahrenheit please test air conditioning also with thermostat controls. |  |  |
|  | ATTIC. |  |  |
|  | No stains on underside of roofing, especially around roof penetrations. |  |  |
|  | Sufficient insulation and properly installed insulation. 10-14 inches deep. |  |  |
|  | Adequate ventilation, clear path into attic for air entering through soffit vents, no insulation blocking vent areas. |  |  |
|  | No plumbing, exhaust or appliance vents terminating in attic. All electrical components sealed. |  |  |
|  | 4 |  |  |